- Linearization

Code:

%% Linearization syms x xdot y ydot z zdot phi theta psi p q r syms u1 u2 u3 u4 syms m g I_x I_y I_z k_x k_y k_z k_p k_q k_r syms z_d psi_d states = [x y z xdot ydot zdot phi theta psi p q r]; inputs = [u1 u2 u3 u4]; equilibrium = [0 0 z_d 0 0 0 0 0 psi_d 0 0 0 m*g 0 0 0]; xddot = (1/m)*((cos(phi)*sin(theta)*cos(psi) + sin(phi)*sin(psi))*u1 - k_x*xdot); yddot = (1/m)*((cos(phi)*sin(theta)*sin(psi) - sin(phi)*cos(psi))*u1 - k_y*ydot); zddot = (1/m)*(cos(phi)*cos(theta)*u1 - m*g - k_z*zdot); phi_dot = p + sin(phi)*tan(theta)*q + cos(phi)*tan(theta)*r; theta_dot = cos(phi)*q - sin(phi)*r; psi_dot = (sin(phi) * q)/cos(theta) + (cos(phi) * r)/cos(theta); pdot = (1/I_x)*((I_y - I_z)*q*r + u2 - k_p*p); qdot = (1/I_y)*((I_z - I_x)*p*r + u3 - k_q*q); rdot = (1/I_z)*((I_x - I_y)*p*q + u4 - k_r*r); sys = [xdot, ydot, zdot, xddot, yddot, zddot, phi_dot, theta_dot, psi_dot, pdot, qdot, rdot]; y = [x y z psi]; Asym = subs(jacobian(sys, states), [states inputs], equilibrium); Bsym = subs(jacobian(sys, inputs), [states inputs], equilibrium); Csym = subs(jacobian(y, states), [states inputs], equilibrium); Dsym = subs(jacobian(y, inputs), [states inputs], equilibrium); parameters = [m g I_x I_y I_z k_x k_y k_z k_p k_q k_r z_d psi_d]; numerics = [0.03 9.81 1.5e-5 1.5e-5 3e-5 4.5e-3 4.5e-3 4.5e-3 4.5e-4 4.5e-4 4.5e-4 2 pi/4]; A = double(vpa(subs(Asym, parameters, numerics))); B = double(vpa(subs(Bsym, parameters, numerics))); C = double(vpa(subs(Csym, parameters, numerics))); D = double(vpa(subs(Dsym, parameters, numerics))); sys = ss(A, B, C, D); TF = tf(sys);

Transfer Functions:

Gzs= 1003s2+0.15s

Gψ(s)=103*104s2+15s

- Stability

Code:

%% Stability [V,D,W] = eig(A); % V consists of the eigenvectors in its columns, D consists of the eigenvalues on its diagonals [V_Jordan, J] = jordan(A); % J is the Jordan form of A

Eigenvalues (on the diagonals):

D=[]

Eigenvectors (each column is an eigenvector associated with the corresponding eigenvalue):

V=[]

Jordan form:

J=[]

The linearized model at the equilibrium is unstable. Because two of the eigenvalues, namely 0, has a zero real part with corresponding Jordan block larger than 1*1. They are J([1 2],[1 2]) = [0 1; 0 0] and J([5 6],[5 6]) = [0 1; 0 0], which show that the model is unstable.

- Forced Response

Code:

%% Forced Response A_heading = [A(9,9) A(9,12); A(12,9) A(12,12)]; B_heading = [B(9,4); B(12,4)]; C_heading = [C(4,9) C(4,12)]; D_heading = 0; sys_heading = ss(A_heading, B_heading, C_heading, D_heading); subplot(3,1,1); [y, t] = impulse(3e-5*sys_heading, 1); plot(t, y, 'LineWidth',2) ylim([0, 0.07]) title("Impulse response", 'FontSize',16); xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); subplot(3,1,2); [y,t] = step(3e-5*sys_heading, 1); plot(t, y, 'LineWidth',2) title("Step Response", 'FontSize',16); xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); data_points = 10000; stoptime = 1; dt = 1/data_points; t = (0:dt:stoptime); u = 3e-5*sin(t); y = lsim(sys_heading, u, t); subplot(3,1,3); plot(t,y, 'Linewidth', 2); title("Sinusoidal input response", 'FontSize',16); xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1);

Plots:

Verification using final value theorem:

Gψ(s)=103*104s2+0.15s

Ys=Gψs*Us=103*104s2+0.15s* 3*10-5*1=1s2+15s

lims→0s*Fs=lims→0 s*1s2+15s= 1/15≈0.0667

The result of final value theorem coincides with the result shown by MATLAB.

- Proportional Control

Code:

%% Proportional Control A_altitude = [A(3,3) A(3,6); A(6,3) A(6,6)]; B_altitude = [B(3,1); B(6,1)]; C_altitude = [1 0]; D_altitude = 0; x0 = [-1;0]; k = -1; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,1) [y, t] = initial(sys_altitude, x0, 20); plot(t,y, 'linewidth',2) title("k=-1: Divergence", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); k = 0; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,2) [y,t]=initial(sys_altitude, x0, 1); plot(t,y, 'linewidth', 2) title("k=0: Converge to non-zero(-1)", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); k = 0.0001; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,3) [y,t]=initial(sys_altitude, x0, 300); plot(t,y, 'linewidth', 2) title("k=0.0001: Converge to zero without oscillation", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); k = 0.01; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,4) [y,t]=initial(sys_altitude, x0, 200); plot(t,y,'linewidth',2) title("k=0.01: Converge to zero with oscillation", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1);

Plots:

Case Divergence: choose k = -1

Case Converge to non-zero: choose k = 0

Case Converge to zero without oscillation: choose k = 0.0001

Case Converge to zero with oscillation: choose k = 0.01

Full MATLAB code:

close all clear all clc %% Linearization syms x xdot y ydot z zdot phi theta psi p q r syms u1 u2 u3 u4 syms m g I_x I_y I_z k_x k_y k_z k_p k_q k_r syms z_d psi_d states = [x y z xdot ydot zdot phi theta psi p q r]; inputs = [u1 u2 u3 u4]; equilibrium = [0 0 z_d 0 0 0 0 0 psi_d 0 0 0 m*g 0 0 0]; xddot = (1/m)*((cos(phi)*sin(theta)*cos(psi) + sin(phi)*sin(psi))*u1 - k_x*xdot); yddot = (1/m)*((cos(phi)*sin(theta)*sin(psi) - sin(phi)*cos(psi))*u1 - k_y*ydot); zddot = (1/m)*(cos(phi)*cos(theta)*u1 - m*g - k_z*zdot); phi_dot = p + sin(phi)*tan(theta)*q + cos(phi)*tan(theta)*r; theta_dot = cos(phi)*q - sin(phi)*r; psi_dot = (sin(phi) * q)/cos(theta) + (cos(phi) * r)/cos(theta); pdot = (1/I_x)*((I_y - I_z)*q*r + u2 - k_p*p); qdot = (1/I_y)*((I_z - I_x)*p*r + u3 - k_q*q); rdot = (1/I_z)*((I_x - I_y)*p*q + u4 - k_r*r); sys = [xdot, ydot, zdot, xddot, yddot, zddot, phi_dot, theta_dot, psi_dot, pdot, qdot, rdot]; y = [x y z psi]; Asym = subs(jacobian(sys, states), [states inputs], equilibrium); Bsym = subs(jacobian(sys, inputs), [states inputs], equilibrium); Csym = subs(jacobian(y, states), [states inputs], equilibrium); Dsym = subs(jacobian(y, inputs), [states inputs], equilibrium); parameters = [m g I_x I_y I_z k_x k_y k_z k_p k_q k_r z_d psi_d]; numerics = [0.03 9.81 1.5e-5 1.5e-5 3e-5 4.5e-3 4.5e-3 4.5e-3 4.5e-4 4.5e-4 4.5e-4 2 pi/4]; A = double(vpa(subs(Asym, parameters, numerics))); B = double(vpa(subs(Bsym, parameters, numerics))); C = double(vpa(subs(Csym, parameters, numerics))); D = double(vpa(subs(Dsym, parameters, numerics))); sys = ss(A, B, C, D); TF = tf(sys) %% Stability [V,D,W] = eig(A); % V consists of the eigenvectors in its columns, D consists of the eigenvalues on its diagonals [V_Jordan, J] = jordan(A);% J is the Jordan form of A %% Forced Response A_heading = [A(9,9) A(9,12); A(12,9) A(12,12)]; B_heading = [B(9,4); B(12,4)]; C_heading = [C(4,9) C(4,12)]; D_heading = 0; sys_heading = ss(A_heading, B_heading, C_heading, D_heading); subplot(3,1,1); [y, t] = impulse(3e-5*sys_heading, 1); plot(t, y, 'LineWidth',2) ylim([0, 0.07]) title("Impulse response", 'FontSize',16); xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); subplot(3,1,2); [y,t] = step(3e-5*sys_heading, 1); plot(t, y, 'LineWidth',2) title("Step Response", 'FontSize',16); xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); data_points = 10000; stoptime = 1; dt = 1/data_points; t = (0:dt:stoptime); u = 3e-5*sin(t); y = lsim(sys_heading, u, t); subplot(3,1,3); plot(t,y, 'Linewidth', 2); title("Sinusoidal input response", 'FontSize',16); xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); %% Proportional Control A_altitude = [A(3,3) A(3,6); A(6,3) A(6,6)]; B_altitude = [B(3,1); B(6,1)]; C_altitude = [1 0]; D_altitude = 0; x0 = [-1;0]; k = -1; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,1) [y, t] = initial(sys_altitude, x0, 20); plot(t,y, 'linewidth',2) title("k=-1: Divergence", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); k = 0; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,2) [y,t]=initial(sys_altitude, x0, 1); plot(t,y, 'linewidth', 2) title("k=0: Converge to non-zero(-1)", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); k = 0.0001; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,3) [y,t]=initial(sys_altitude, x0, 300); plot(t,y, 'linewidth', 2) title("k=0.0001: Converge to zero without oscillation", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1); k = 0.01; A_controlled_altitude = A_altitude - k*B_altitude*C_altitude; sys_altitude = ss(A_controlled_altitude, [], C_altitude, []); subplot(2,2,4) [y,t]=initial(sys_altitude, x0, 200); plot(t,y,'linewidth',2) title("k=0.01: Converge to zero with oscillation", 'Fontsize',16) xlabel("Time(seconds)", 'FontSize',14) ylabel("Amplitude", 'FontSize',14) set(gca,'linewidth',1);