- Lead Controller

(a) The lead controller is chosen with parameters:

k=110000

T=40

α=0.15

The resulting lead controller is:

Ks=11000040s+16s+1

(b) The Bode plot is attached below.

(c) The closed-loop step response is attached below.

(d) It’s not possible to achieve all four specifications. As shown in the open-loop Bode plot of the uncompensated transfer function (attached below), at frequency 4 rad/s, the phase is less than -240°. To achieve a phase margin of at least 40° while having the gain crossover frequency of at least 4 rad/s, a phase lead of more than 100° has to be introduced at the frequency of at least 4 rad/s, which is impossible for a single lead compensator. A single lead compensator can provide a phase lead of only 90° asymptotically. So, requirement III and requirement IV are contradicting and cannot be achieved simultaneously. The lead compensator designed above achieves requirements I, II, and IV.

- Cascade Control

(a)

Gouts=49.05s(s+23)(s+5)

(b)

k1=1, α1=0.1, T1=1→Kout,1s=s+10.1s+1

k2=120, α2=13, T1=3→Kout,2s=1203s+133s+1

(c) The overlaid Bode plots are attached below.

(d) The overlaid closed-loop step responses are attached below.

(e) Using stepinfo(), the table is filled below.

| Controller | Kout,1s | Kout,2s | No Cascade Control |
| --- | --- | --- | --- |
| Rise Time(s) | 0.2012 | 1.7320 | 17.1033 |
| % Overshoot | 78.4441 | 4.9654 | 36.3413 |

- Model Mismatch

(a)

| Controller | Kout,1s | Kout,2s |
| --- | --- | --- |
| Maximum delay Td(ms) | 26.0 | 1505.8 |

(b) The step responses with delay of 200ms is attached below. Since 200ms>26.0ms, which exceeds the maximum delay allowed for controller 1, the closed-loop system becomes unstable and the step response diverges. While for the controller 2, it’s within the range of allowed delay. The closed-loop system is still stable and the step response converges. The results of step responses agree with the maximum delay analysis.

(c)

Serrs=Gout(s)1+Kout(s)Gout(s)=49.05ss+23s+5+49.05Kout

(d)

αlag=40, Tlag=10→Klags=4010s+1400s+1

(e) The overlaid step responses with measurement error for both uncompensated and compensated plants are attached below. As shown in the plot, the compensated controller 2 rises to within 5cm error at 18.3s. And its steady state error is much smaller than using the controller 2, and the steady state error is even smaller than using the uncompensated controller 1 with much less oscillation.

(f) The Bode plot of Klags*Kout,2s is attached below. As shown in the plot, the phase margin is 55.1°, which is above 55°.

Full MATLAB code:

%% Lead Compensator G_CF = tf(9.81, [1 2/3 0 0]); K = tf([40/10000 1/10000], [40*0.15 1]); margin(G_CF) margin(K*G_CF) step(feedback(K*G_CF,1),500) %% Cascade Control g_theta = tf(1,[1 0]); tf_theta = feedback(5*g_theta,1); G_no_theta = tf(9.81, [1 2/3 0]); G_out = tf_theta * G_no_theta; % Crossover @>=4rad/s K1 = tf([1 1],[1*(0.1) 1]); % bode(K1*G_out); % step(feedback(K1*G_out,1)); % stepinfo(feedback(K1*G_out,1)); % Phase margin >= 60 degree K2 = tf([sqrt(3)/20 1/20],[1/3*sqrt(3) 1]); % bode(K2*G_out); % step(feedback(K2*G_out,1)); % stepinfo(feedback(K1*G_out,1)); margin(K1*G_out) hold on margin(K2*G_out) hold off title("Phase Margin_1=8.46^{\circ}, w_{c1}=5.68rad/s Phase Margin_2=62.9^{\circ}, w_{c2}=0.729rad/s") step(feedback(K1*G_out,1),20); hold on step(feedback(K2*G_out,1),20); legend('K_{out,1}','K_{out,2}') stepinfo(feedback(K*G_CF,1)) stepinfo(feedback(K1*G_out,1)) stepinfo(feedback(K2*G_out,1)) %% Model Mismatch % Max delay [Gm1,Pm1,Wcg1,Wcp1] = margin(K1*G_out); max_time_delay1 = deg2rad(Pm1)/Wcp1; [Gm2,Pm2,Wcg2,Wcp2] = margin(K2*G_out); max_time_delay2 = deg2rad(Pm2)/Wcp2; % Step response with delay tf_delay = tf(1,1,'InputDelay',0.2); step(feedback((K1*G_out),tf_delay),10) ylim([-5,5]) hold on step(feedback((K2*G_out),tf_delay),10) ylim([-5,5]) hold off title("Step response with delay") legend('K_{out,1}','K_{out,2}') %S error syms x S1_num = sym2poly(49.05*((7-4*sqrt(3))*x+1)); S1_den = sym2poly(x*((7-4*sqrt(3))*x+1)*(x+2/3)*(x+5) +49.05*(x+1)); S1 = tf(S1_num, S1_den); S2_num = sym2poly(49.05*(sqrt(3)/3*x+1)); S2_den = sym2poly(x*(sqrt(3)/3*x+1)*(x+2/3)*(x+5)+(49.05/20)*(sqrt(3)*x+1)); S2 = tf(S2_num, S2_den); %Step response with error T1 = feedback(K1*G_out,1); tf_error_1 = T1 - pi/180*S1; T2 = feedback(K2*G_out,1); tf_error_2 = T2 - pi/180*S2; % Lag Compensator K_lag = tf([400 40],[400 1]); S_lag_num = sym2poly(49.05*(sqrt(3)/3*x+1)*(400*x+1)); S_lag_den = sym2poly((sqrt(3)/3*x+1)*(400*x+1)*x*(x+2/3)*(x+5)+(49.05/20)*(40)*(sqrt(3)*x+1)*(10*x+1)); S_lag = tf(S_lag_num, S_lag_den); T_lag = feedback(K_lag*K2*G_out,1); tf_error_lag = T_lag - pi/180*S_lag; step(tf_error_1,30) hold on step(tf_error_2,30) hold on step(tf_error_lag,30) title("Step Response with measurement error") legend('K_{out,1}','K_{out,2}','K_{lag}*K_{out,2}') hold off margin(K_lag*K2*G_out)