- Lead Controller

(a) The lead controller is chosen with parameters:

k=110000

T=40

α=0.15

The resulting lead controller is:

Ks=11000040s+16s+1

(b) The Bode plot is attached below.

(c) The closed-loop step response is attached below.

(d) It’s not possible to achieve all four specifications. As shown in the open-loop Bode plot of the uncompensated transfer function (attached below), at frequency 4 rad/s, the phase is less than -240°. To achieve a phase margin of at least 40° while having the gain crossover frequency of at least 4 rad/s, a phase lead of more than 100° has to be introduced at the frequency of at least 4 rad/s, which is impossible for a single lead compensator. A single lead compensator can provide a phase lead of only 90° asymptotically. So, requirement III and requirement IV are contradicting and cannot be achieved simultaneously. The lead compensator designed above achieve requirements I, II, and IV.

- Cascade Control

(a)

Gouts=49.05s(s+23)(s+5)

(b)

k1=1, α1=0.1, T1=1→Kout,1s=s+10.1s+1

k2=120, α2=13, T1=3→Kout,2s=1203s+133s+1

(c) The overlaid Bode plots are attached below.

(d) The overlaid closed-loop step responses are attached below.

(e) Using stepinfo(), the table is filled below.

| Controller | Kout,1s | Kout,2s | No Cascade Control |
| --- | --- | --- | --- |
| Rise Time | 0.2012 | 1.7320 | 17.1033 |
| % Overshoot | 78.4441 | 4.9654 | 36.3413 |

- Model Mismatch

(a)

| Controller | Kout,1s | Kout,2s |
| --- | --- | --- |
| Maximum delay Td | 26.0ms | 1505.8ms |

(b) The step responses with delay of 200ms is attached below. Since 200ms>0.0260s, which exceeds the maximum delay allowed for controller 1, the closed-loop system becomes unstable and the step response diverges. While for the controller 2, it’s within the range of allowed delay. The closed-loop system is still stable and the step response converges. The results of step responses agree with the maximum delay analysis.

(c)

Serrs=Gout1+KoutGout=49.05ss+23s+5+49.05Kout

(d)

αlag=40, Tlag=10→Klags=4010s+1400s+1

(e) The overlaid step responses with measurement error for both uncompensated and compensated plants are attached below. As shown in the plot, the compensated controller 2 rises to within 5cm error at 18.3s. And its steady state error is smaller than using the uncompensated controller 1 with much less oscillation.

(f) The Bode plot of Klags*Kout,2s is attached below. As shown in the plot, the phase margin is 55.1°, which is above 55°.

Full MATLAB code:

%% Lead Compensator G_CF = tf(9.81, [1 2/3 0 0]); K = tf([40/10000 1/10000], [40*0.15 1]); margin(G_CF) margin(K*G_CF) step(feedback(K*G_CF,1),500) %% Cascade Control g_theta = tf(1,[1 0]); tf_theta = feedback(5*g_theta,1); G_no_theta = tf(9.81, [1 2/3 0]); G_out = tf_theta * G_no_theta; % Crossover @>=4rad/s K1 = tf([1 1],[1*(0.1) 1]); % bode(K1*G_out); % step(feedback(K1*G_out,1)); % stepinfo(feedback(K1*G_out,1)); % Phase margin >= 60 degree K2 = tf([sqrt(3)/20 1/20],[1/3*sqrt(3) 1]); % bode(K2*G_out); % step(feedback(K2*G_out,1)); % stepinfo(feedback(K1*G_out,1)); margin(K1*G_out) hold on margin(K2*G_out) hold off title("Phase Margin_1=8.46^{\circ}, w_{c1}=5.68rad/s Phase Margin_2=62.9^{\circ}, w_{c2}=0.729rad/s") step(feedback(K1*G_out,1),20); hold on step(feedback(K2*G_out,1),20); legend('K_{out,1}','K_{out,2}') stepinfo(feedback(K*G_CF,1)) stepinfo(feedback(K1*G_out,1)) stepinfo(feedback(K2*G_out,1)) %% Model Mismatch % Max delay [Gm1,Pm1,Wcg1,Wcp1] = margin(K1*G_out); max_time_delay1 = deg2rad(Pm1)/Wcp1; [Gm2,Pm2,Wcg2,Wcp2] = margin(K2*G_out); max_time_delay2 = deg2rad(Pm2)/Wcp2; % Step response with delay tf_delay = tf(1,1,'InputDelay',0.2); step(feedback((K1*G_out),tf_delay),10) ylim([-5,5]) hold on step(feedback((K2*G_out),tf_delay),10) ylim([-5,5]) hold off title("Step response with delay") legend('K_{out,1}','K_{out,2}') %S error syms x S1_num = sym2poly(49.05*((7-4*sqrt(3))*x+1)); S1_den = sym2poly(x*((7-4*sqrt(3))*x+1)*(x+2/3)*(x+5) +49.05*(x+1)); S1 = tf(S1_num, S1_den); S2_num = sym2poly(49.05*(sqrt(3)/3*x+1)); S2_den = sym2poly(x*(sqrt(3)/3*x+1)*(x+2/3)*(x+5)+(49.05/20)*(sqrt(3)*x+1)); S2 = tf(S2_num, S2_den); %Step response with error T1 = feedback(K1*G_out,1); tf_error_1 = T1 - pi/180*S1; T2 = feedback(K2*G_out,1); tf_error_2 = T2 - pi/180*S2; % Lag Compensator K_lag = tf([400 40],[400 1]); S_lag_num = sym2poly(49.05*(sqrt(3)/3*x+1)*(400*x+1)); S_lag_den = sym2poly((sqrt(3)/3*x+1)*(400*x+1)*x*(x+2/3)*(x+5)+(49.05/20)*(40)*(sqrt(3)*x+1)*(10*x+1)); S_lag = tf(S_lag_num, S_lag_den); T_lag = feedback(K_lag*K2*G_out,1); tf_error_lag = T_lag - pi/180*S_lag; step(tf_error_1,30) hold on step(tf_error_2,30) hold on step(tf_error_lag,30) title("Step Response with measurement error") legend('K_{out,1}','K_{out,2}','K_{lag}*K_{out,2}') hold off margin(K_lag*K2*G_out)